MANGALORE REFINERY AND PETROCHEMICALS LIMITED

P-2104B Return-to-Service Assessment

CDU-2 rotating equipment / prepared for the morning review

Recommendation

Pump P-2104B must not be returned to service on its existing seal arrangement. Vibration has risen from 3.9 mm/s to 9.6 mm/s over eight weeks, crossing the 7.1 mm/s alert threshold on 2026-08-13 [VIB-CDU2-2026_Vibration_Log.xlsx, sheet Readings].

Evidence

Overall velocity reached 9.6 mm/s on 2026-08-27, which is 87% of the 11.0 mm/s trip limit [VIB-CDU2-2026_Vibration_Log.xlsx, sheet Limits].

The August seal failure on the same machine was attributed to Plan 52 pot level loss [INC-2026-0042_Seal_Failure_P-2104B.pdf, p.2].

HAZOP action H7-03 covering seal-pot low-level alarms remains OVERDUE [HAZOP-CDU2-2024_Node7_Action_Register.pdf, p.1].

Blockers

Seal kit SK-682-2104-A3 shows zero on hand with a 45-day lead time [SPR-ROT-001_Spares_Inventory.xlsx, sheet Stock].

Hot work in CDU-2 is suspended until 2026-11-30 under circular SC-2026-11 [SC-2026-11_Hot_Work_Restriction_CDU2.pdf, p.1].

Two operators have lapsed competency for the changeover SOP [TRN-CDU2-MATRIX_Competency_Matrix.xlsx, sheet Matrix].

Next steps

Raise an expedite request against the seal kit, close H7-03 before restart, and run the changeover on P-2104A per SOP-CDU2-014 Rev 4 step 7 [SOP-CDU2-014_Rev4_Pump_Changeover.pdf, p.3].

PROVENANCE AND CHAIN OF CUSTODY

| Generated | 2026-09-03 20:14:13 (on-premise) |
| --- | --- |
| Model | qwen2.5:7b-instruct served locally at http://127.0.0.1:11434 |
| Question | Can we return P-2104B to service, and what is blocking the seal upgrade? |
| Sources cited | 6 document reference(s) |
| Audit chain | 29 records, intact, head e827297318776cb6cd24bfcc90b12de4 |
| Session | 4194ed5e9f57 |

Sources

VIB-CDU2-2026_Vibration_Log.xlsx, sheet Readings

INC-2026-0042_Seal_Failure_P-2104B.pdf, p.2

HAZOP-CDU2-2024_Node7_Action_Register.pdf, p.1

SPR-ROT-001_Spares_Inventory.xlsx, sheet Stock

SC-2026-11_Hot_Work_Restriction_CDU2.pdf, p.1

SOP-CDU2-014_Rev4_Pump_Changeover.pdf, p.3

Generated on-premise by the Sovereign AI Workbench. No data left this machine. Source documents are synthetic prototype data.